SECTION 10 31 00

MANUFACTURED GAS FIREPLACES

Display hidden notes to specifier. (Don't know how? Click Here)

** NOTE TO SPECIFIER ** Stellar by Heat & Glo is a brand and product line of Hearth & Home Technologies.

which is located at:
7571 215th Street W

Lakeville, MN 55044
Tel: 952-224-4072
Fax: 952-224-4074
Email: stellarinfo@hearthnhome.com
Web: https://www.stellar.heatnglo.com
[Click Here] for additional information.
GENERAL

SECTION INCLUDES

** NOTE TO SPECIFIER ** Delete items below not required for project.

Gas fireplaces of the following types:

The Enlight Collection™ manufactured gas fireplace by Stellar by Heat & Glo

- (NOTE: This product line was formerly called the Foundation Series & Galaxy Series)

RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete any sections below not relevant to this project; add others as required.

Section 06 10 00 - Rough Carpentry.

Section 21 20 00 - Fire-Extinguishing Systems.

Section 26 05 00 - Common Work Results for Electrical.

REFERENCES

Underwriters Laboratories (UL):

UL 127 - Standard for Factory-Built Fireplaces.

American National Standards Institute (ANSI)

ANSI Z21.50-2016 – Vented Decorative Gas Appliances

ANSI Z223.1/NFPA 54 – National Fuel Code

Canadian Standards Association (CSA)

CSA 2/22-2016 – Vented Decorative Gas Appliances

SUBMITTALS

Submit under provisions of Section 01 30 00 - Administrative Requirements.

Product Data:

Manufacturer's data sheets on each product to be used.

Preparation instructions and recommendations.

Storage and handling requirements and recommendations.

Typical installation methods.

** NOTE TO SPECIFIER ** Delete selection samples if colors have already been selected.

Selection Samples: For each finish product specified, two complete sets of color chips representing manufacturer's full range of available colors and patterns.

Verification Samples: For each finish product specified, two samples, minimum size 6 inches (150 mm) square representing actual product, color, and patterns.

Shop Drawings: Include details of materials, construction, and finish. Include relationship with adjacent construction.

Installation Manual - Please consult the manufacturer’s installation manual for all details and requirements before making a final design layout decision.

QUALITY ASSURANCE

Manufacturer Qualifications: Company specializing in manufacturing products specified in this section with a minimum five years documented experience.

Installer Qualifications: Company specializing in performing Work of this section with minimum two years documented experience with projects of similar scope and complexity.

Source Limitations: Provide each type of product from a single manufacturing source to ensure uniformity.

** NOTE TO SPECIFIER ** Include mock-up if the project size or quality warrant the expense. The following is one example of how a mock-up on might be specified. When deciding on the extent of the mock-up, consider all the major different types of work on the project. Delete if not required.

Mock-Up: Construct a mock-up with actual materials in sufficient time for Architect's review and to not delay construction progress. Locate mock-up as acceptable to Architect and provide temporary foundations and support.

Intent of mock-up is to demonstrate quality of workmanship and visual appearance.

If mock-up is not acceptable, rebuild mock-up until satisfactory results are achieved.

Retain mock-up during construction as a standard for comparison with completed work.

Do not alter or remove mock-up until work is completed or removal is authorized.

PRE-INSTALLATION CONFERENCE

Convene a conference approximately two weeks before scheduled commencement of the Work. Attendees shall include Architect, Contractor and trades involved. Agenda shall include schedule, responsibilities, critical path items and approvals.

DELIVERY, STORAGE, AND HANDLING

Store and handle in strict compliance with manufacturer's written instructions and recommendations.

Protect from damage due to weather, excessive temperature, and construction operations.

PROJECT CONDITIONS

Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's recommended limits.

SEQUENCING

Ensure that products of this section are supplied to affected trades in time to prevent interruption of construction progress.

WARRANTY

Manufacturer's Warranty: Provide manufacturer's standard limited warranty.

- PRODUCTS

MANUFACTURERS

Acceptable Manufacturer: Stellar by Heat & Glo®, which is located at: 7571 215th St. W.; Lakeville, MN 55044; Toll Free Tel: 952-224-4072; Email: stellarinfo@hearthnhome.com Web: www.stellar.heatnglo.com

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

Substitutions: N/A.

Requests for substitutions will be considered in accordance with provisions of Section 01 60 00 - Product Requirements.

MANUFACTURED GAS FIREPLACES

Basis of Design: Enlight Collection™ as manufactured by Stellar by Heat & Glo

UL Certified gas fireplace.

Burner: Ribbon burner.

Control: On/off switch.

Control: Remote Control

Glazing: Safe-T-Touch glass.

Exhaust: Powervent/B-Vent System.

LED Accent Lighting

** NOTE TO SPECIFIER ** Select configuration required. Delete configurations not required.

Configuration: Left Corner

Configuration: Right Corner

** NOTE TO SPECIFIER ** Select interior panels required. Black is standard. Four-Sided configuration does not have interior panels. Delete options not required.

Interior Panels: Black.

Interior Panels: Mirrored.

Interior Panel:  Custom Envision™ Panel (requires additional lead time)

** NOTE TO SPECIFIER ** Select media options required. Delete options not required.

Media: Glass, 1/2 to 3/4 inch (13 to 19 mm) size.

Color: Crystal (Ships Standard)

Color: Ebony

Color: Amber

Color: Sea Breeze

Color: Amber

Color: Copper

Color: Dark Blue

Media: Natural Stone.

Stone: Polished Gold

Stone: Polished Black

Stone: Polished Black (Large)

Stone: Polished White

Stone: Polished Red

Stone: Polished Mixed

Stone: Lava Stone

** NOTE TO SPECIFIER ** Log set is optional. Delete if not required.

Log Set: Driftwood Log Set.

** NOTE TO SPECIFIER ** Universal louver is optional. Delete if not required.

Louver: Universal intake and exhaust louver.

** NOTE TO SPECIFIER ** Select model required. Delete models not required.

Model: Enlight Corner 3 ft x 20 in

BTU/Hour Input: 40,000

Exhaust B-Vent: 8”

Outside Air: 10”

RS Series Power Vent: RS012

In-Line Power Vent: RSIF160

Finishing Width Front: N/A

Finishing Height: 20 [508]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 38 [1,067]

Overall Height: 46 [1,168]

Center Of Vent: 20-7/8 [530]

Firebox Depth: 24-1/32 [610]

Overall Width: 41-3/4 [1,065]

Minimum Framing Width: 42-3/4 [1,086]

Minimum Framing Height: 52 [1,321]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 3 ft x 24 in

BTU/Hour Input: 40,000

Exhaust B-Vent: 8”

Outside Air: 10”

RS Series Power Vent: RS012

In-Line Power Vent: RSIF160

Finishing Width Front: 39 31/32 [1,015]

Finishing Height: 24 [610]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 42 [1,067]

Overall Height: 50 [1,270]

Center Of Vent: 20-7/8 [530]

Firebox Depth: 24-1/32 [610]

Overall Width: 41-3/4 [1,065]

Minimum Framing Width: 42-3/4 [1086]

Minimum Framing Height: 56 [1,422]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 3 ft x 30 in

BTU/Hour Input: 40,000

Exhaust B-Vent: 8”

Outside Air: 10”

RS Series Power Vent: RS012

In-Line Power Vent: RSIF160

Finishing Width Front: 39-31/32 [1015]

Finishing Height: 30 [762]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 48 [1,219]

Overall Height: 56 [1,422]

Center Of Vent: 20-7/8 [530]

Firebox Depth: 24-1/32 [610]

Overall Width: 41-3/4 [1,065]

Minimum Framing Width: 42-3/4 [1086]

Minimum Framing Height: 62 [1,574]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 3 ft x 36 in

BTU/Hour Input: 40,000

Exhaust B-Vent: 8”

Outside Air: 10”

RS Series Power Vent: RS012

In-Line Power Vent: RSIF160

Finishing Width Front: 39-31/32 [1015]

Finishing Height: 36 [914]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 54 [1,372]

Overall Height: 62 [1,575]

Center Of Vent: 20-7/8 [530]]

Firebox Depth: 24-1/32 [610]

Overall Width: 41-3/4 [1,065]

Minimum Framing Width: 42-3/4 [1,086]

Minimum Framing Height: 68 [1,727]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 3 ft x 48 in

BTU/Hour Input: 40,000

Exhaust B-Vent: 8”

Outside Air: 10”

RS Series Power Vent: RS012

In-Line Power Vent: RSIF160

Finishing Width Front: 39-31/32 [1015]

Finishing Height: 48 [1,219]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 66 [1,676]

Overall Height: 74 [1,880]

Center Of Vent: 20-7/8 [530]

Firebox Depth: 24-1/32 [610]

Overall Width: 41-3/4 [1,065]

Minimum Framing Width: 42-3/4 [1,086]

Minimum Framing Height: 80 [2,032]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 3 ft x 60 in

BTU/Hour Input: 40,000

Exhaust B-Vent: 8”

Outside Air: 10”

RS Series Power Vent: RS012

In-Line Power Vent: RSIF160

Finishing Width Front: 39-31/32 [1015]

Finishing Height: 60 [1,521]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 78 [1,981]

Overall Height: 86 [2,184]

Center Of Vent: 20-7/8 [530]

Firebox Depth: 24-1/32 [610]

Overall Width: 41-3/4 [1,065]

Minimum Framing Width: 42-3/4 [1,086]

Minimum Framing Height: 92 [2,337]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 4 ft x 20 in

BTU/Hour Input: 60,000

Exhaust B-Vent: 8”

Outside Air: 10”

RS Series Power Vent: RS012

In-Line Power Vent: RSIF160

Finishing Width Front: 51-31/32 [1320]

Finishing Height: 20 [508]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 38 [1,067]

Overall Height: 46 [1,168]

Center Of Vent: 26-7/8 [683]

Firebox Depth: 24-1/32 [610]

Overall Width: 53-3/4 [1,365]

Minimum Framing Width: 54-3/4 [1,391]

Minimum Framing Height: 52 [1,321]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 4 ft x 24 in

BTU/Hour Input: 60,000

Exhaust B-Vent: 8”

Outside Air: 10”

RS Series Power Vent: RS012

In-Line Power Vent: RSIF160

Finishing Width Front: 51-31/32 [1320]

Finishing Height: 24 [610]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 42 [1,067]

Overall Height: 50 [1,270]

Center Of Vent: 26-7/8 [683]

Firebox Depth: 24-1/32 [610]

Overall Width: 53-3/4 [1,365]

Minimum Framing Width: 54-3/4 [1,391]

Minimum Framing Height: 56 [1,422]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 4 ft x 30 in

BTU/Hour Input: 60,000

Exhaust B-Vent: 8”

Outside Air: 10”

RS Series Power Vent: RS012

In-Line Power Vent: RSIF160

Finishing Width Front: 51-31/32 [1320]

Finishing Height: 30 [762]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 48 [1,219]

Overall Height: 56 [1,422]

Center Of Vent: 26-7/8 [683]

Firebox Depth: 24-1/32 [610]

Overall Width: 53-3/4 [1,365]

Minimum Framing Width: 54-3/4 [1,391]

Minimum Framing Height: 62 [1,574]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 4 ft x 36 in

BTU/Hour Input: 60,000

Exhaust B-Vent: 8”

Outside Air: 10”

RS Series Power Vent: RS012

In-Line Power Vent: RSIF160

Finishing Width Front: 51-31/32 [1320]

Finishing Height: 36 [914]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 54 [1,372]

Overall Height: 62 [1,575]

Center Of Vent: 26-7/8 [683]

Firebox Depth: 24-1/32 [610]

Overall Width: 53-3/4 [1,365]

Minimum Framing Width: 54 -3/4 [1391]

Minimum Framing Height: 68 [1,727]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 4 ft x 48 in

BTU/Hour Input: 60,000

Exhaust B-Vent: 8”

Outside Air: 10”

RS Series Power Vent: RS012

In-Line Power Vent: RSIF160

Finishing Width Front: 51-31/32 [1320]

Finishing Height: 48 [1,219]

Finishing Width End: 22- 3/16[564]

Height To Trim Top: 66 [1,676]

Overall Height: 74 [1,880]

Center Of Vent: 26-7/8 [683]

Firebox Depth: 24-1/32 [610]

Overall Width: 53-3/4 [1,365]

Minimum Framing Width: 54-3/4 [1391]

Minimum Framing Height: 80 [2,032]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 4 ft x 60 in

BTU/Hour Input: 60,000

Exhaust B-Vent: 8”

Outside Air: 10”

RS Series Power Vent: RS012

In-Line Power Vent: RSIF160

Finishing Width Front: 51-31/32 [1320]

Finishing Height: 60 [1,521]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 78 [1,981]

Overall Height: 86 [2,184]

Center Of Vent: 26-7/8 [683]

Firebox Depth: 24-1/32 [610]

Overall Width: 53-3/4 [1,365]

Minimum Framing Width: 54-3/4 [1,391]

Minimum Framing Height: 92 [2,337]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 5 ft x 20 in

BTU/Hour Input: 80,000

Exhaust B-Vent: 8”

Outside Air: 10”

RS Series Power Vent: RS012

In-Line Power Vent: RSIF160

Finishing Width Front: 63-31/32 [1625]

Finishing Height: 20 [508]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 38 [1,067]

Overall Height: 46 [1,168]

Center Of Vent: 32-7/8 [835]

Firebox Depth: 24-1/32 [610]

Overall Width: 65-3/4 [1,670]

Minimum Framing Width: 66 -3/4 [1695]

Minimum Framing Height: 52 [1,321]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 5 ft x 24 in

BTU/Hour Input: 80,000

Exhaust B-Vent: 8”

Outside Air: 10”

RS Series Power Vent: RS012

In-Line Power Vent: RSIF160

Finishing Width Front: 63-31/32 [1625]

Finishing Height: 24 [610]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 42 [1,067]

Overall Height: 50 [1,270]

Center Of Vent: 32-7/8 [835]

Firebox Depth: 24-1/32 [610]

Overall Width: 65-3/4 [1,670]

Minimum Framing Width: 66-3/4 [1,695]

Minimum Framing Height: 56 [1,422]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 5 ft x 30 in

BTU/Hour Input: 80,000

Exhaust B-Vent: 8”

Outside Air: 10”

RS Series Power Vent: RS012

In-Line Power Vent: RSIF160

Finishing Width Front: 63-31/32 [1625]

Finishing Height: 24 [610]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 48 [1219]

Overall Height: 56 [1422]

Center Of Vent: 32-7/8 [835]

Firebox Depth: 24-1/32 [610]

Overall Width: 65-3/4 [1,670]

Minimum Framing Width: 66-3/4 [1,695]

Minimum Framing Height: 62 [1574]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 5 ft x 36 in

BTU/Hour Input: 80,000

Exhaust B-Vent: 8”

Outside Air: 10”

RS Series Power Vent: RS012

In-Line Power Vent: RSIF160

Finishing Width Front: 63-31/32 [1625]

Finishing Height: 36 [914]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 54 [1372]

Overall Height: 62 [1575]

Center Of Vent: 32-7/8 [835]

Firebox Depth: 24-1/32 [610]

Overall Width: 65-3/4 [1,670]

Minimum Framing Width: 66-3/4 [1,695]

Minimum Framing Height: 68 [1727]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 5 ft x 48 in

BTU/Hour Input: 80,000

Exhaust B-Vent: 8”

Outside Air: 10”

RS Series Power Vent: RS012

In-Line Power Vent: RSIF160

Finishing Width Front: 63-31/32 [1625]

Finishing Height: 48 [1219]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 66 [1676]

Overall Height: 74 [1880]

Center Of Vent: 32-7/8 [835]

Firebox Depth: 24-1/32 [610]

Overall Width: 65-3/4 [1,670]

Minimum Framing Width: 66-3/4 [1,695]

Minimum Framing Height: 80 [2032]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 5 ft x 60 in

BTU/Hour Input: 80,000

Exhaust B-Vent: 8”

Outside Air: 10”

RS Series Power Vent: RS012

In-Line Power Vent: RSIF160

Finishing Width Front: 63-31/32 [1625]

Finishing Height: 60 [1,521]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 78 [1,981]

Overall Height: 86 [2,184]

Center Of Vent: 32-7/8 [835]

Firebox Depth: 24-1/32 [610]

Overall Width: 65-3/4 [1,670]

Minimum Framing Width: 66-3/4 [1695]

Minimum Framing Height: 92 [2,337]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 6 ft x 20 in

BTU/Hour Input: 100,000

Exhaust B-Vent: 10”

Outside Air: 10”

RS Series Power Vent: RS014

In-Line Power Vent: RSIF180

Finishing Width Front: 75-31/32 [1930]

Finishing Height: 20 [508]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 38 [1,067]

Overall Height: 46 [1,168]

Center Of Vent: 38-7/8 [987]

Firebox Depth: 24-1/32 [610]

Overall Width: 77-3/4 [1,975]

Minimum Framing Width: 78-3/4 [2000]

Minimum Framing Height: 52 [1,321]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 6 ft x 24 in

BTU/Hour Input: 100,000

Exhaust B-Vent: 10”

Outside Air: 10”

RS Series Power Vent: RS014

In-Line Power Vent: RSIF180

Finishing Width Front: 75-31/32 [1930]

Finishing Height: 24 [610]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 42 [1,067]

Overall Height: 50 [1,270]

Center Of Vent: 38-7/8 [987]

Firebox Depth: 24-1/32 [610]

Overall Width: 77-3/4 [1,975]

Minimum Framing Width: 78-3/4 [2000]

Minimum Framing Height: 56 [1,422]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 6 ft x 30 in

BTU/Hour Input: 100,000

Exhaust B-Vent: 10”

Outside Air: 10”

RS Series Power Vent: RS014

In-Line Power Vent: RSIF180

Finishing Width Front: 75-31/32 [1930]

Finishing Height: 30 [762]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 48 [1,219]

Overall Height: 56 [1,422]

Center Of Vent: 38-7/8 [987]

Firebox Depth: 24-1/32 [610]

Overall Width: 77-3/4 [1,975]

Minimum Framing Width: 78-3/4 [2000]

Minimum Framing Height: 62 [1,574]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 6 ft x 36 in

BTU/Hour Input: 100,000

Exhaust B-Vent: 10”

Outside Air: 10”

RS Series Power Vent: RS014

In-Line Power Vent: RSIF180

Finishing Width Front: 75-31/32 [1930]

Finishing Height: 36 [914]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 54 [1,372]

Overall Height: 62 [1,575]

Center Of Vent: 38-7/8 [987]

Firebox Depth: 24-1/32 [610]

Overall Width: 77-3/4 [1,975]

Minimum Framing Width: 78-3/4 [2000]

Minimum Framing Height: 68 [1,727]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 6 ft x 48 in

BTU/Hour Input: 100,000

Exhaust B-Vent: 10”

Outside Air: 10”

RS Series Power Vent: RS014

In-Line Power Vent: RSIF180

Finishing Width Front: 75-31/32 [1930]

Finishing Height: 48 [1,219]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 66 [1,676]

Overall Height: 74 [1,880]

Center Of Vent: 38-7/8 [987]

Firebox Depth: 24-1/32 [610]

Overall Width: 77-3/4 [1,975]

Minimum Framing Width: 78-3/4 [2000]

Minimum Framing Height: 80 [2,032]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 6 ft x 60 in

BTU/Hour Input: 100,000

Exhaust B-Vent: 10”

Outside Air: 10”

RS Series Power Vent: RS014

In-Line Power Vent: RSIF180

Finishing Width Front: 75-31/32 [1930]

Finishing Height: 60 [1,521]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 78 [1,981]

Overall Height: 86 [2,184]

Center Of Vent: 38-7/8 [987]

Firebox Depth: 24-1/32 [610]

Overall Width: 77-3/4 [1975]

Minimum Framing Width: 78-3/4 [2000]

Minimum Framing Height: 92 [2,337]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 7 ft x 20 in

BTU/Hour Input: 120,000

Exhaust B-Vent: 10”

Outside Air: 10”

RS Series Power Vent: RS014

In-Line Power Vent: RSIF180

Finishing Width Front: 87-31/32 [2234]

Finishing Height: 20 [508]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 38 [1,067]

Overall Height: 46 [1,168]

Center Of Vent: 44-7/8 [1,140]

Firebox Depth: 24-1/32 [610]

Overall Width: 89-3/4 [2,280]

Minimum Framing Width: 90-3/4 [2205]

Minimum Framing Height: 52 [1,321]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 7 ft x 24 in

BTU/Hour Input: 120,000

Exhaust B-Vent: 10”

Outside Air: 10”

RS Series Power Vent: RS014

In-Line Power Vent: RSIF180

Finishing Width Front: 87-31/32 [2234]

Finishing Height: 24 [610]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 42 [1,067]

Overall Height: 50 [1,270]

Center Of Vent: 44-7/8 [1,140]

Firebox Depth: 24-1/32 [610]

Overall Width: 89-3/4 [2,280]

Minimum Framing Width: 90-3/4 [2205]

Minimum Framing Height: 56 [1,422]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 7 ft x 30 in

BTU/Hour Input: 120,000

Exhaust B-Vent: 10”

Outside Air: 10”

RS Series Power Vent: RS014

In-Line Power Vent: RSIF180

Finishing Width Front: 87-31/32 [2234]

Finishing Height: 30 [762]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 48 [1,219]

Overall Height: 56 [1,422]

Center Of Vent: 44-7/8 [1,140]

Firebox Depth: 24-1/32 [610]

Overall Width: 89-3/4 [2,280]

Minimum Framing Width: 90-3/4 [2205]

Minimum Framing Height: 62 [1,574]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 7 ft x 36 in

BTU/Hour Input: 120,000

Exhaust B-Vent: 10”

Outside Air: 10”

RS Series Power Vent: RS014

In-Line Power Vent: RSIF180

Finishing Width Front: 87-31/32 [2234]

Finishing Height: 36 [914]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 54 [1,372]

Overall Height: 62 [1,575]

Center Of Vent: 44-7/8 [1,140]

Firebox Depth: 24-1/32 [610]

Overall Width: 89-3/4 [2,280]

Minimum Framing Width: 90-3/4 [2205]

Minimum Framing Height: 68 [1,727]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 7 ft x 48 in

BTU/Hour Input: 120,000

Exhaust B-Vent: 10”

Outside Air: 10”

RS Series Power Vent: RS014

In-Line Power Vent: RSIF180

Finishing Width Front: 87-31/32 [2234]

Finishing Height: 48 [1,219]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 66 [1,676]

Overall Height: 74 [1,880]

Center Of Vent: 44-7/8 [1,140]

Firebox Depth: 24-1/32 [610]

Overall Width: 89-3/4 [2,280]

Minimum Framing Width: 90-3/4 [2205]

Minimum Framing Height: 80 [2,032]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 7 ft x 60 in

BTU/Hour Input: 120,000

Exhaust B-Vent: 10”

Outside Air: 10”

RS Series Power Vent: RS014

In-Line Power Vent: RSIF180

Finishing Width Front: 87-31/32 [2234]

Finishing Height: 60 [1,521]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 78 [1,981]

Overall Height: 86 [2,184]

Center Of Vent: 44-7/8 [1,140]

Firebox Depth: 24-1/32 [610]

Overall Width: 89-3/4 [2,280]

Minimum Framing Width: 90-3/4 [2205]

Minimum Framing Height: 92 [2,337]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 8 ft x 20 in

BTU/Hour Input: 140,000

Exhaust B-Vent: 10”

Outside Air: 12”

RS Series Power Vent: RS014

In-Line Power Vent: RSIF180

Finishing Width Front: 99-31/32 [2539]]

Finishing Height: 20 [508]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 40 [1,016]

Overall Height: 52 [1,321]

Center Of Vent: 50-7/8 [1,321]

Firebox Depth: 24-1/32

Overall Width: 101-3/4 [2,584]

Minimum Framing Width: 102-3/4 [2610]

Minimum Framing Height: 58 [1,473]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 8 ft x 24 in

BTU/Hour Input: 140,000

Exhaust B-Vent: 10”

Outside Air:12”

RS Series Power Vent: RS014

In-Line Power Vent: RSIF180

Finishing Width Front: 99-31/32 [2539]

Finishing Height: 24 [610]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 44 [1,118]

Overall Height: 56 [1,422]

Center Of Vent: 50-7/8 [1,321]

Firebox Depth: 24-1/32 [610]

Overall Width: 101-3/4 [2,584]

Minimum Framing Width: 102-3/4 [2610]

Minimum Framing Height: 62 [1,574]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 8 ft x 30 in

BTU/Hour Input: 140,000

Exhaust B-Vent: 10”

Outside Air: 12”

RS Series Power Vent: RS014

In-Line Power Vent: RSIF180

Finishing Width Front: 99-31/32 [2539]

Finishing Height: 30 [762]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 50 [1,270]

Overall Height: 62 [1,575]

Center Of Vent: 50-7/8 [1,321]

Firebox Depth: 24-1/32 [610]

Overall Width: 101-3/4 [2,584]

Minimum Framing Width: 102-3/4 [2610]

Minimum Framing Height: 68 [1,727]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 8 ft x 36 in

BTU/Hour Input: 140,000

Exhaust B-Vent: 10”

Outside Air: 12”

RS Series Power Vent: RS014

In-Line Power Vent: RSIF180

Finishing Width Front: 99-31/32

Finishing Height: 36 [914]

Finishing Width End: 22-3/16

Height To Trim Top: 56 [1,422]

Overall Height: 68 [1,727]

Center Of Vent: 50-7/8 [1,321]

Firebox Depth: 24-1/32 [610]

Overall Width: 101-3/4 [2,584]

Minimum Framing Width: 102-3/4 [2610]

Minimum Framing Height: 74 [1,880]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 8 ft x 48 in

BTU/Hour Input: 140,000

Exhaust B-Vent: 10”

Outside Air: 12”

RS Series Power Vent: RS014

In-Line Power Vent: RSIF180

Finishing Width Front: 99-31/32 [2539]

Finishing Height: 48 [1219]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 68 [1,727]

Overall Height: 80 [2032]

Center Of Vent: 50-7/8 [1,321]

Firebox Depth: 24-1/32 [610]

Overall Width: 101-3/4 [2,584]

Minimum Framing Width: 102-3/4 [2610]

Minimum Framing Height: 86 [2184]

Minimum Framing Depth: 24-1/2 [622]

Model: Enlight Corner 8 ft x 60 in

BTU/Hour Input: 140,000

Exhaust B-Vent: 10”

Outside Air: 12”

RS Series Power Vent: RS014

In-Line Power Vent: RSIF180

Finishing Width Front: 99-31/32 [2,539]

Finishing Height: 60 [1,521]

Finishing Width End: 22-3/16 [564]

Height To Trim Top: 80 [2,032]

Overall Height: 92 [2,337]

Center Of Vent: 50-7/8 [1,321]

Firebox Depth: 24-1/32 [610]

Overall Width: 101-3/4 [2,584]

Minimum Framing Width: 102-3/4 [2,610]

Minimum Framing Height: 98 [2,489]

Minimum Framing Depth: 24-1/2 [622]

- EXECUTION

EXAMINATION

Do not begin installation until substrates have been properly constructed and prepared.

If substrate preparation is the responsibility of another installer, notify Architect in writing of unsatisfactory preparation before proceeding.

PREPARATION

Clean surfaces thoroughly prior to installation.

Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

INSTALLATION

Install in accordance with manufacturer's instructions, approved submittals and in proper relationship with adjacent construction.

Use manufacturer's guidelines for minimum clearances to combustibles, walls, and finishes.

FIELD QUALITY CONTROL

Upon completion of installation, visually inspect all exposed surfaces. Touch up scratches and abrasions as recommended by the manufacturer.

Test for proper operation, control and safety devices.

CLEANING AND PROTECTION

Clean products in accordance with the manufacturer’s recommendations.

Touch-up, repair or replace damaged products before Substantial Completion.

Protect installed products until completion of project.

END OF SECTION